Cellcop Communicator
CP72u-M
August 2012
User Manual

[image: image1.png]



ABOUT THE CELLCOP COMMUNICATOR SYSTEM

The Cellcop GSM communicator system is based on SMS and GPRS technology. It uses standard cellphone technology for communication and has been designed to provide you with the greatest possible flexibility and convenience. Read this manual carefully and have your installer instruct you on your system’s operation and on which features have been implemented in your system. All users of this system should be equally instructed in its use. 

INDEX
1. Features

2. Getting Started
2.1 Description
2.2 Typical hardware requirements
2.3 The CP72u-M board layout
2.4 The CP72u-M configuration software layout
2.5 Installing the configuration software

2.6 Connecting the CP72u-M board to a PC / Laptop via USB cable.

2.6.1 Installing the USB Driver

2.6.1.1 Driver Installation on Windows XP

2.6.1.2 Driver Installation on Windows 7

2.6.2 Setting up the configuration software to connect to the communicator board via USB cable. 
2.7 Setting up the configuration software to connect to the communicator using a GSM Modem (Remote configuration)
2.8 Backing up configuration detail to a file and loading it back again.
3. Configuring the communicator
3.1 Hardware Connections overview

3.2 Cellphone numbers and names
3.3 Power Inputs and statuses
3.4 Digital Inputs
3.5 Digital Outputs
3.6 Scheduler

3.7 Monitor mode
3.8 System parameters and data call connection

3.9 GPRS setup
3.10 Setting RTC and testing board

3.11 USB Port

3.12 Antenna

3.13 SIM Card

4. Controlling the GSM Communicator using a Cellphone

4.1 SMS Control Commands to communicator

4.1.1 Custom commands 

4.1.2 Status request commands

4.1.2.1 Main status request

4.1.2.2 Hour meter status request

4.1.2.3 Counter Status Request

4.1.2.4 Properties

4.2 SMS Programming Commands to communicator

4.2.1 Set Real Time Clock (RTC)

4.2.2 Set hour meter value

4.2.3 Set counter value

4.2.4 Set Hour meter Limit for maintenance

4.2.5 Schedule commands
5. Applications
6. Troubleshooting
1. Features

7 Inputs to communicate separate alarm conditions

· Each input can be triggered to send an SMS to up to 16 Cellphone numbers

· The time delay before the input is triggered can be set for each input

· Separate messages can be configured for On and Off states of the input signal

· Messages to be send can be programmed by the user

· On or Off states can both be reported to predefined cellphone numbers.

· Reporting can be disabled for an input

· The states of the inputs can be requested from the unit by SMS

· Monitoring of the inputs can be controlled using different modes

· Inputs can act as counters
· Inputs can act as Run Hour meters

2 Outputs to control any electrical device

· Outputs can be controlled by cellphone using SMS (Switching the output on, off or pulse)

· Output controls can be scheduled

· The duration of the pulse can be programmed for each output

· Outputs can be set to follow the state of an input

· Output can be set to switch on when the unit is dialed

· Status of an output can be requested from the unit by SMS

· Analog Limits can control the switching of outputs

· Own text can be used to control outputs

Monitor AC power using the charger input 

· AC power can be monitored by using the charger input.

· SMS can be send to up to 16 numbers when an power failure occur and when the power return

Monitor the battery status 

· The battery status can be monitored by the system.

· SMS can be send to up to 16 numbers when the battery go faulty.

· The battery is monitored by disconnecting it from the main supply and to measure the battery voltage while connected to a load.

· Add and delete numbers via SMS

Scheduled Commands

· The execution of commands can be scheduled

Monitor Mode
· Monitor mode is used to activate monitor functions
· Monitor Mode can be set by SMS or Monitor mode Inputs
Configuration tool to configure unit to user requirements

· All parameters can be set from configuration tool.

· Remote programming possible from the configuration tool

· Save and Load from files for backup

2. Getting Started
2.1 Description
The cellcop communicator can be used in various applications to monitor and control system alarms and parameters remotely using your cellphone. The unit can be configured to your requirements by using the configuration tool. The configuration tool allows the user to setup various options by setting the parameters and sending the setup to the cellcop. Using the configuration tool u can setup the Cellcop directly using a USB cable connected to the to the cellcop or remotely by connecting a GSM modem to the PC running the configuration tool and connect to the cellcop using a data call. The configuration can be backed up to file using the configuration tool for easy reuse in future or quickly setting up multiple boards with the same configuration.
2.2 Typical hardware requirements

For a typical installation you will require the following:

· CP72u-M Communicator board
[image: image2.png]



· SIM card (Remember that the card must be a RICA card)

· 
The card must have been registered on RICA for use in South Africa (South Africa only)

· 
In case of a pre-paid card it must have sufficient funds or SMS bundles loaded

· 
The card must be packed switched data enabled in case of GPRS use

· 
The card must be enabled for Circuit Switch data for remote programming via data call.
[image: image3.png]



· Power, Inputs and outputs harness connector

Can be bought from your supplier
[image: image4.png]



· GSM Antenna
Supplied by yourself or by your supplier
	[image: image5.png]




	[image: image6.png]





· Power supply

Supplied by yourself or by GSM Systems

Power supply
14.2V  DC ( 5%
· Relay board if unit will be used to switch external equipment

Can be bought from your supplier
[image: image7.jpg]NT72C-s10
o1y’





1 relay add-on

· PC running configuration software and USB cable

Hardware can be bought from GSM Systems

Software available on CD from GSM Systems or can be downloaded from

http://www.drivehq.com/file/df.aspx/publish/cellcop/CellcopFiles/CP72u-M
2.3 The CP72u-M board layout

Harness and USB side zoomed view

[image: image8.png]S e }

USB Port
Configuration





SIM and antenna side zoomed view
[image: image9.png]




2.4 The CP8844u-M configuration software layout

[image: image10.png]Tl P L ACM Cuctorme Vor Fina 20117 TR TTE A NREm N,
[ CP7x2u-M Configuration Tool - Copyright GSM Systems Ver: June 2012 =

Dig

Power

Addtional Functions

System Setup

Testing

Delay before tigger (122 sec Units]: 1
‘SHS text o send on trigger or CounteriHour Meter Prefix

8 10 11 1213 14 15 16
EEEEEEEN

Tnput 1 Off - (Contact Open) I Enavk liode  (on) B Enabie iods A (077 Il Enabi iode 5 (Skep.

Delay before tigger (122 sec Units]: 1 =
‘SHS text 1o send on trigger or CounteriHour Meter Suffoc —
Input 1 0fF

8 10 11 1213 14 15 16

Counter 1

Enable put 1 Counter Mutipler 1000000 3] ung Counts

P Counton Ciose. Count on Delayed Ciose:

Run/Hour Meter 1

Enavl nput 1 Hourliter yunpier 1000000 (2] unt Fours Enav Servoe SUS il
(RUN on Close)

1000 Hours.
Sencetmt 190 5

Load from File Write to file ~Read from Cellcop 1 |
@)





(The color and number of the blocks drawn on the diagram represent the description below)

1. Area to set up numbers to report to

2. Setting up communications to the communicator board and connecting to it
3. Loading and writing configuration files to and from hard disk

4. Main functionality selection tab

5. Secondary selection tab to select sub-sections for the main functionality tab

6. Configuration information for single entities as selected from main functionality tab and the secondary tab.
7. Reading current configuration and writing new configuration to the cellcop communicator

2.5 Installing the configuration software

The configuration tool is a normal executable and can be copied to your own location from the supplied CD or from the internet (See link below). Run the Configuration Tool from the location where you copied it to. 

The newest version can be downloaded from:

http://www.drivehq.com/file/df.aspx/publish/cellcop/CellcopFiles/CP72u-M
2.6 Connecting the CP8844u-M board to a PC / Laptop via USB cable.

To set up the communicator with the functionality required from a PC or Laptop you have to connect the communicator board to the PC or Laptop with an USB cable. Then you have to use the configuration tool software package on your PC or Laptop to set up the board. A few things have to be set up and connected for this process. We will discuss setting up the USB driver on the PC or Laptop and copying the required configuration software onto the PC or Laptop for later use.
2.6.1 Installing the USB Driver
To use the USB programming option u need to install the USB driver. You can find the driver on the supplied CD or by downloading it from
http://www.drivehq.com/file/df.aspx/publish/cellcop/CellcopFiles/CP72u-M
2.6.1.1 Driver Installation on Windows XP

When the USB cable is inserted for the first time a window will popup to begin the driver installation.
[image: image11.jpg]Welcome to the Found New
Hardware Wizard
“This wizard helps you instal software for

USBto UART

(@), ¥ yur hbwore come wits annstllaon CD
< or floppy disk. insert it now.

What do you want the wizard to do?

" Instll the software automaticaly (Recommended)
@ Install from a st o specifclocation (Advanced)

Cick Next o continue.

e





Select Install from specific location and click next.

[image: image12.jpg]Browse F

ider (7]

Select the foder that contains crivers for your harduiare.

3 Datashests -
122 Dealers.

3 Delohi =
= 9 Drivertss
D vsta

& Homesuddy -

‘ 3
To view any subfolders, cick 2 plus sign above.

o ] e |





Select the location where the driver is located. Driver directory on the supplied CD.

[image: image13.jpg]Found New Hardware Wizard

Please choose your search and installation options.

& Search fothe best diver i these locations.

Use the check boxes below to it or expand the defauit search, which includss focal
paths and removable media. The best diver found wil be nstalled

I Search removable meda floppy, CD-ROM. )
¥ Incude this locaton i the search:

£ Documerts nd Setings Admn PAL Deskiop C <] [_Browse

 Dont search. | il choose the diverto nstal.

Choose this option to sefect the devioe criverfrom a s, Windows does not quarantee that
the diver you choose wil be the best match for your hardware.





Click Next

[image: image14.jpg]Hardware Installation

‘ The softuare you ae nstalin fo i arre:
Ussio UART

has not passed Windows Logo testing to verfy s compatbily
Wi Windows XP. (Tell me why this esting s mportant

Continuing your installation of this software may impair
or destabilize the correct operation of your system
either immediately orin the future. Microsoit strongly
recommends that you stop this installation now and
contact the hardware vendor for software that has
passed Windows Logo testing.

s oy | [ |





Click Continue Anyway

[image: image15.jpg]Completing the Found New
Hardware Wizard

The wizard has fiished instaling the softwars for

(3 USBto UART





Click Finish. Driver is successfully installed. This will add a serial port to your existing ports. Use this serial port when configuring the unit.
To find out which port is the USB port, go to the device manager and click on ports. It’s the port with USB to UART next to it. Use this port when configuring the unit.
[image: image16.jpg]Fle Acton View Hep

s 2m A =g

=8 omeC
3 Computer
< Dskdrves
 Diply adapters
DVD/CD-ROM drives
S Floppy dsk controers
@8 Human Interface Devices
B DEATA/ATAPL controlers
S Keyboards
%mmmmmm
MirodipTooks
@ Moritors
<> Multfnction adapters
B Network adapters
= 5 Portabie Deviees
8 Nokia ES0
=¥ Ports (comaLeT)
7 Commuricaions Port (coM1)
7 Lantronix R port (CoM12)
7 e seralport o)
7 P SeralPort coma)
7 Printer port (PT1)
gwwmzm
5 48 Frocessors
@), Sound, video and game controllers.
System devices
Uriversal Seria Bus controlers





2.6.1.2 Windows 7 Driver Installation

Windows 7 will install the wrong driver when inserted the first time. To install the correct driver go to the Device manager. A icon showing USB serial demo should be visible. Right click in this icon and select update Driver.

[image: image17.jpg]How do you want to search for driver software?

+ Search automatically for updated driver software
Windows il sesrch your computer and the Interet or the atest drver software
foryour device, unles you've disabled this esture n your device nsallstion
sttings.

» Browse my computer for driver software
Locste and instll driver software manualy.




 

Select browse Computer for driver. Go to location where the driver is stored on your hard disk or on the supplied CD under the driver directory.

[image: image18.jpg]Browse for driver software on your computer

Search for driver software i this location:

2 Let me pick from a list of device drivers on my computer
This st will show instlled driver softwsre compatible with the device, and sl drver
softwsre in the same category as the device.





Select the directory for the driver of the version you have (32 or 64Bit)
[image: image19.jpg]Select the foder that contains crivers for your hardare.





[image: image20.jpg]@m0 s & @&

T JohanPCWT

57" combcom - seial port emulators

5488 Computer

5 s Diskdrves

» By Display adapters

-3 DVD/CD-ROM drives

-l Human Interface Devices

- IDE ATA/ATAPI contrllers:

525 Imaging devices

52 Keyboards

5§ Mice and other pointing devices

-] MicrochipTools

-4 Modems

5 Monitors

52 Network adapters

477 Ports (COM & LPT)
Y% Communications Port (COML)
5 HHD Software Virtual Serial Port (COMS3)
5 HHD Software Virtual Serial Port (COMS4)
Y3 High-Speed PCl Serial Port (COM3).
Y% High-Speed PCl Serial Port (COM4)
77 USB to UART (COMT) |

» I Processors

5+ Sound, video and game controlers

5488 System devices

5§ Universal Seia Bus controlers





The driver should be installed. Note the port next to the USB to Uart line. This will be used when configuring the unit in the future.
2.6.2 Setting up the configuration software to connect to the communicator board. 
[image: image21.jpg]W P00 Confgration Tool- Copyght GSM Systems Ver27 My 212 ==
/= Cellphone Numbers

h.,““.,...n_,angngEEEIEEI

01 o2 03 04 05 05 o7 OB

e wo_|| e

Communications

—

- & maiE el AlE Tl Rl o





We need to set up the serial port that the CP72u-M communicator is connected to on the configuration tool software. Click “Setup” Button on the left bottom of the configuration tool window. The following screen will appear:
[image: image22.jpg]COM port configuration

ot
e

Stopbits parity
@1 15 2 © None Mark
B odd Space
4 05 @6 O7 @8 ) Even
Flow control
None © DTRRTS XonXoff





Change the port to the port where the communicator is connected. Don’t change anything else. Click OK when finished.

Click on the “Open Port” Button to enable the communications

[image: image23.png]=~
1 omsSevelCell] 1 Namei |

2 cdhoez |2 meme2 |
3 copones 3 s | d
4 coornes |4 emes |13
5 Coores |5 vmes (| O
6  Celphones 6 Nemes g
7 celphone7 7 name7 &
8 s 18 s |
9 coes 19 s | B
10  celphone 10 10 teme 10 3
N e A1 wemen | B
12 celphone 12 12 wmez | g
13 cobtores |13 emen || &
14 Celprone 14 14 neme 14 2
15 Celphone 15 15 omess |
16 Celphone 16 16 Memets |

= Cellphone Numbers

Signal T |





Click the “Read from Cellcop” button to read the information from the GSM communicator. The progress bar will progress from 0 to 100 % as the information is read from the communicator. If the progress bar gets stuck at 5 % then you selected the wrong port or the configuration cable is not connected or the configuration cable is faulty.
Change the information and parameters to fit your setup

Click “Write to Cellcop” button to write your new configuration to the unit

The unit is now ready for normal operation.
2.7 Setting up the configuration software to connect to the communicator using a GSM Modem (Remote configuration)

Remote configuration using a GSM Modem use the same actions as the USB configuration but communications needs to be established with the remote unit first.

GSM Modem

	[image: image24.png]



	


NB: Please note that a remote connection can’t be made to a board that is connected via USB to a PC. The USB connection takes privilege.

Connect a GSM modem to your computer. Note the Communication port for the GSM modem. 

Select the communications port by clicking on the SETUP Button and follow instructions as described in 2.6.1 in this manual.

If the modem connection is open then click on the System Setup Tab, it is second from the bottom on the vertical tabs. You will have the following TAB open.
[image: image25.png]I

Ste Adress / Name / Number: Country Code:
A SNSteNiame 27
2 Programming Password Control Password,
z 12345 67890
H

Remote Cellcop Cellphone Number SETUP MODEM

SIS Message or Hodem Command

Connect to Remote Cellcop oot poscae

Disconnect Remote Cellcop

Testing

(Ach'vate Data Communis ti(‘ms) (Send SMS) (Selld Command )





So let’s open the connection.

[image: image26.png]Communications




Click on “Open Port” if successful the open port button will background color will become green and the button text will change to “Close Port”. 

[image: image27.png]Communications

el |




You might see text in the middle screen come up like:

[image: image28.png]Di

Digital Outputs.

Power

Testing

e

Ste Adress / Name / Number: Country

SNSteNiame 27

Programming Password

12345 67890

rov
<cru: 1
<CPI: READY
pacse 1

CallReady

Remote Cellcop Cellphone Number
2miizase

Connect to Remote Cellcop

Disconnect Remote Cellcop

Code:

Control Password,

SETUP MODEM

SIS Message or Hodem Command

Test Message

(Ach'vate Data Cmnmlmi(zti(‘ms)

(send sms}  (‘send command |





Enter the remote Cellcop’s number and click on ‘Connect to remote Cellcop”. Wait until you see Connect 9600. If nothing happen make sure you have a GSM Modem connected to the PC and that it is switched on and the port is open. You are now connected and need to authorize the connection. Click on Activate data connection. Make sure your Programming password and the remote units programming password match. If they match you will get a message indicating communications is allowed. Now you can read and write configuration information to the communicator as well as download SD card logs (on the Additional Functions / SD Card Logger Screen). You can also SET the Date and time on the Testing Page (on the TAB below the current one).

Please note that you should click on “Disconnect remote Cellcop” button and then the “Close Port” Button when you completed your remote session with the communicator board so that the board can return to normal operation.
2.8 Backing up configuration detail to a file and loading it back again.
Backing up your configuration to a file is an essential way of making sure your future changes and installations of the same application become easier. To backup the configuration you first need to get the configuration into the configuration tool by reading from the Cellcop by clicking on the “Read from Cellcop” button or by changing the parameters directly on the tool. Then you click on the “write to file” button and supply the file name for the configuration backup. A nice method is to use the site name and date as part of you file name. That will make it identifiable in the future. This is just a guide you can setup your own name if you like.

To restore the configuration from a backed up file you must click on the “Load from file” button. Then you write to Cellcop to send the configuration to the Cellcop by clicking on the “Write to Cellcop” button.

3. Configuring the communicator
The cellcop communicator is designed to be used in various industries and applications. The user can define configuration parameters to suit the requirements for the application that they need the Cellcop communicator for. The configuration tool is used to define the specific parameters and to load the configuration into the Cellcop. A copy of the parameters can also be saved to file for future use or as a backup. The following sections will explain the various features and layout of the Cellcop and how to configure each feature.
3.1 Hardware Connections overview
Front View
[image: image29.png]



The bottom has the harness connector and USB port. [image: image30.png]S e }

USB Port
Configuration





The top has the screw on port for the antenna and also the SIM card holder is on the top side of the communicator
[image: image31.png]



Please note that the communicator must have a SIM card to connect to the network. For good reception it has to be fitted with a GSM antenna. It has to have a power supply to operate. If it is used as a logger it should have a Micro SD card. The LCD screen is optional.

3.2 Cellphone Numbers and names                 
Alarms and status messages can be sent to specified cell numbers. These numbers must be set up on the communicator. Please note the position numbers where the cell numbers are entered. These positions are selected or de-selected in other configuration screens to select whether messages are sent or not sent to these numbers. You can enter up to 16 numbers.

If a GPRS server is used enter the server number into position number 1.

The number is entered on the left side and you can enter a description on the right.

NB: Numbers must be entered in the international format. Replace 0 with country code. Example: 0825436789 must be +27825436789 on South African Networks.
[image: image32.jpg]/= Cellphone Numbers

1 ammnr 1

2 wmus 2 o
3 Celphones 3 names
4 Celphones 4 hemes
5 Cebhones | 5 Names
6 cebhones | B Names
7 celphone7 7 vame7
8 celphones 8 names
9 Celphones 9 hames
10 Ceprone0 | 100 Neme10
11 Coprone1i | A1 Nemeir
12 Cephone2 | 12 Nemei2
13 Ceohones | 13 Neme13
14 Copronet4 | 14 Name1d
15 Cephones | 15 Nemeis
16 Cephones | 16 Nemets

-





3.3 Power Inputs and statuses
The Power Inputs has 5 Pins. This port is used to power the unit and to supply backup power. There is also 1 Vout pins to supply output power for sensors. The output voltage is a function of the input voltage. A maximum of 0.5 A can be drawn from these outputs. Find it at the right hand side bottom of the unit.
[image: image33.png]S e }

USB Port
Configuration





	
	


[image: image34.png]



We can set up alarms on power failures and monitor charging and backup battery statuses. Select the Power tab on the vertical tabs.
In1
-
Yellow/Red

In2
-
Brown
In3
-
Grey


In4
-
Blue/Grey
In5
-
Green


In6
-
Blue/Yellow
In7
-
White/Grey

Out2
-
Yellow
Tx
-
Blue


Rx
-
White
3V3
-
Orange

GND 
- 
Black

Vout
-
Red


Out1
-
Purple

Vin
-
Red


GND 
- 
Black

[image: image35.png]Digital Inputs.

Analog Inputs

Digital Outputs

‘Additional Functions

System Setup

Testing

M Encie Hode N (Ol Enable Hode A (077 Bl Enabe Mode B (Skcp)

Delay before trigger 112 sec Unis}: 5]

111 1213 14 15 16

Log change to On Event  Log Alarm On Event (Delay)
(m] (m]

I Encie Hode N (Ol Enable Hode A (077 Il Enabe Mode B (Skep)

Delay before trigger [12 sec Units]: 1 5]
Add St Name.

|

111 1213 14 15 16

Log change to OffEvent  Log Alarm Off Event (Delay)
(m] (m]

Battery Status M Enatie hode (o)l Enable ode A (017 Il Enabl Hode B (Seep)

Low /Fauty Batery Levet: 278 [ 1000V
SHS textto send on Battery LowFauty: Arnes
Batiery Low/Fauty 0

12 3 45 6 7 808 1011 1213 14 15 16
Sl EEEEEEEEEENEEENEN

L Log change to On Event
GPRS Only. ol




You can enter the message to be sent on mains power on / off and / or battery status. The sent to: indicates the cell numbers that the message must be sent to. If all is selected as above screen shows it will still only send to numbers that are set up. So if position 3 to 16 has no cell numbers defined it will only send the message to cell numbers in position 1 and 2.

The delay, before the trigger, work in half second units. So if you want the message only to be send after the power has been off for 10 seconds you set the delay trigger to 20. This is helpful to filter out power dips and not real power outages.

You can set the alarm message only to be sent via GPRS only. This means that it is logged to a database on the internet and not sent to a cellphone via SMS. 

You can also select if the event will be logged and if the event is logged if it is logged when the state change happens or after the timeout has run out after the state change.

You can select on which modes the event should be monitored. For power monitoring the default is on in all modes.

You can enter the level you want to use for testing if the battery has reached a low power level. Typically this will be set to 10V on a 12 V battery.

The add site name option allows you to concatenate the site name of the site sending the message to the back of the SMS message been send.
3.4 Digital Inputs

Digital Inputs is used to report alarms on an On / Off state. The Input is On when the input is connected to ground and off when it is disconnected. The unit can be configured to report both the On and the Off or only a single state. A delay timer can be setup to delay the state change from On to Off and from Off to On. Inputs can also be used as counters and hour meters.

Input events diagram

[image: image36.png]I
'/Send SMS on Input Off

1 H . '
] ] :
1 | /Send SMSon Input On,
P :
Input State| 1 1
Change : 1
: ‘
1 1 1
Time
::>! o :<:' Delay :>! o !<:' Delay
' '
o
g Event after
Input | time delay
1

i

Time line

Log Change Event
without time dalay

T
XLog Change Event

without time dalay

. Log Event after
time delay




The diagram above shows the difference between change events and delayed change events on the inputs. The input state change signal shows the effect after the delay, on On and Off events, has run out. So you can see that the short off period on the Input is ignored by the state changes as it were not long enough for the input state change to be affected. An example of using this functionality is for instance on power monitoring where we only want to report on constant power loss and not power dips. So the power off input must be off for longer than the off timer defined, before the input state change will reflect it, and for instance send an alarm SMS on the power lost input. SMS messages is sent on state changes if activated to send SMS messages in the configuration

The digital inputs are number In1 to In7 and any of the two available grounds (Black) can be used

The inputs are designed to monitor a contact or a GND signal. The input is activated when a GND signal is presented at the input. The input is deactivated when the GND signal is removed from the input.

A typical connection diagram showing connections of a switch on digital input 1 and 2 and a relay switch on digital input 3
[image: image37.png]Supplied Connector with 15cm crimped wire




	
	


We can set up alarms on input changes and monitor these changes and statuses. Select the “Digital Inputs” tab on the vertical tabs.

Configuring Inputs using the configuration tool:
[image: image38.png]Digital Outputs

Power

Additional Functions

System Setup

Testing

[

Input2 ~Input3 Input4 Input5 Input6  Input7

Tnput 1 On - (Contact Closed) B ggavk tode t (on) B enabi iods A (077 Il Enabi iode 5 (Skep.

Deloybefoe rigger [12 sec Unis) 1 =
Add steName

ot 0n
s o s 6 @)

‘SHS text o send on trigger or CounteriHour Meter Prefix

Tnput 1 Off - (Contact Open) MM Enavk liode  (on) B Enabi iods A (077 Il Enabl iode 5 (e,

Delay before tigger [172 sec Urits]: 1 =

‘SHS text 1o send on trigger or CounteriHour Meter Suffoc S —

Input 1 0fF

12 3 45 6 7 808 1011 1213 14 15 16
sl EEEEEEEEEENEENNEN

RS Ony
Counter 1 (9)
Enable nput 1 Counter Mutiplr 1000000 2} uni Counts
» Couton Cose CountonDeeyed Cose
Run/Hour Meter 1
Enal npu 1 ourliter  ytpr 10000 [ un rours Enae Servie SUS il
N an Gose)

000 [= 1000 Hours

L = %





1. To setup parameters on digital inputs select the “Digital Inputs” tab on the vertical tabs.

2. There are 7 digital inputs available and each can be configured separately. 

3. Simply select the input on the top horizontal tabs that you want to configure. You can enter the message to be sent on the On or Off state of the digital input.  The sent to: indicates the cell numbers that the message must be sent to. 

4. If all is selected as above screen shows it will still only send to numbers that are set up. So if position 3 to 16 has no cell numbers defined it will only send the message to cell numbers in position one and 2.

5. The delay, before the trigger, work in half second units. So if you want the message only to be send after the change has been effect for 10 seconds you set the delay trigger to 20. This is helpful to filter out bounce effects on the inputs.

6. You can set the alarm or message only to be sent via GPRS only. This means that it is logged to a database on the internet and not sent to a cellphone via SMS. 

7. You can select on which modes the input should be monitored. For instance a house alarm may be off in Mode A and thus no monitoring of these inputs will happen in Mode A if not ticked. 

8. The add site name option allows you to concatenate the site name of the site sending the message to the back of the SMS message been send.

9. All inputs can have a counter associated with it. Simply enable the counter by ticking its box. Set the multiplier and Units that you want to use for the counter and select if it must count on the input change to ON or on Input delayed state change to on. For instance connecting to a Kw/h meter that gives a pulse for every 0.001 Kw/h used we will set the multiplier to 0.001 and the unit to Kw/h. Then after 100 pulses have been counted we will know that 1 Kw/h has been used.

10. Every input can also have a run meter associated with it. Simply enable the run meter by ticking its box. Set the multiplier and units that you want to use for the run meter. For instance monitoring a pump, pumping water at a rate of 300 cubic liters of water a hour, we will set the multiplier to 300 and set the unit to cubic Liter. By selecting the “ENABLE SERVICE M HM1” option the communicator will send a service message every time the service limit value is reached. This means that if you for instance monitor a Generator that should have a service every 250 hours you can set the value to 250 and check the “ENABLE SERVICE M HM1” option. When 250 hours is reached the communicator will send a service notification SMS. When the service has been done the service personnel can update the value to 500 Hours. So when the Generator has done another 250 hours and reached the total running hours of 500 it will send the next service notification SMS. The process then simply repeats itself.

For examples of applications please see point 5 in the manual
3.5 Digital outputs

Outputs are used to control things remotely.  The outputs on cellcop is designed to switch a 12 V DC relay. On the communicator we have 3 commands per output. The output can be switched On, Off or Pulsed (On for a defined period of time). Various options (SMS, Scheduler, etc.) to trigger these commands on outputs exist.

Connecting a relay board to the outputs

[image: image39.png]Connecting an output

NT72C-510




The outputs are designed to switch a 12 DC relay. The output terminals use 2 pins. Pin 1 is a permanent 12V output, while pin 2 is a switched negative.

Output relay  board:

[image: image40.jpg]NT72C-s10
o1y’




            

1 relay add-on





SMS and Scheduler control

Output state will be controlled by SMS and the scheduler. To switch on an output you must SMS the unit with the text programmed for that function. In the example outputs 1 will switch on when *Out1On# is sent via SMS to the unit from a authorized cellphone

Configuring digital outputs. 

Select the Digital Outputs TAB and then the TAB off the output you want to configure.

[image: image41.png]Digital Inputs.

Power

dditional Functions

System Setup

Testing

Enable SHS Text Control
]

on Oution Pulse OutiPusse

oft outtort

Rlow selecied numbers to control OUtpUL 1
|

6 58 1011

Output 1 Puse ime [Seconds] 1 =
@cabe  SMS /Missed Call / Scheduler Control Output 1

Enable Missed cal

1203 48 7
EEEEEEEEEEEENEENN

Volcs Fax  Data | Enable Scheduer
e |

P Puise Output 1 on missed cal

‘Toggle Output 1 on missed cal
Reply with Status SIS

‘SHS containing Control Password alowed to control Output 1

16





Any pulse command that is executed will pulse for the duration as entered in the output 1 Pulse time [Seconds] field right at the top of the TAB.

[image: image42.png]‘Output 1 Pulse time [Seconds] 20|







SMS and Scheduler control.

Output state will be controlled by SMS and the scheduler. To switch on an output you must SMS the unit with the text programmed for that function. In the example outputs 1 will switch on when *Out1On# is sent via SMS to the unit from an authorized cellphone or a message containing the programming password (cpw*12345#) as well.

[image: image43.png]@ Enable SMS / Missed Call / Scheduler Control Output 1

Enae S5 Text Contl TR = i e e
® n® o o)
Onowion  puse outtpuse I P Caput 1 n s i
o autor > Toook Quiput 1 on missed cat
Repl wih Staus S
5]

SHS contaning Control Passwrd sowed o controlOuput 1
e ~O
7 59 0 M 213 115 16

EEEEEEEENEN




A. To switch on an output you must SMS the unit with the text programmed for that function. In the example output 1 will switch ON when *Out1On#, OFF when *Out1Off# and will pulse for 20 seconds when *Out1Pulse# is sent via SMS to the unit from an authorized cellphone. The controlling text can be changed for the configuration to make more sense to the user like: “*Pump On#”, *Alarm Off#”, *Pulse output#”, etc. The length of the pulse is also configurable in seconds in the output 1 Pulse time [Seconds] field on the top of the window.

B. The: allow selected numbers must be selected to enable any cellphones selected to control the output. In the example all numbers have control rights.

C. If SMS containing Control Password is allowed on output 1 is checked then any cellphone can control the output as long as the password is added to the SMS message been send to the controller. Example of SMS text: cpw*12345# *out1On#
D. Enable scheduler option lets you enable the output for control by scheduled commands.
E. Enable missed call allows you to control the output with a missed voice call. Select whether the missed call will pulse the output or toggle the output If you want a status message back once the output has switched, select the Reply with Status SMS check box.
3.6 Scheduler

Select “Additional Functions” TAB and then the “Scheduler” TAB in the configuration tool to get to the scheduler configuration page. Scheduled commands can be used to schedule a command, Mode change, SMS status, Enable logger, etc. based on a date/time. “77” in any date or time field ignore that field in matching. Setting the date and time to “77” will execute the command every minute for the days of the week that is enabled. 

[image: image44.png]| [ sveduter |~ vontorvece
Tf Date. Weekday Time CMD Date Weekday Time CMD
2 OOMIYYpy Tw T F S 5 HAMM CC DOMIYY |, /7 ¢ 5 5 HMM CC
ot e 777777 ] 0z00 AN 70T M o0 A
H 02 777777 EEEEEE 1500 AF 18- 777777 EEEEEEE 0z00
S|| o3 200000 EMMEMEE 2100 cn 19777777 MMEMEEE o500 A
2{| osp rrocos W W] e op 20777777 MEEEEEE o500 A
o5 777 WU TT L] oan0 A 27 A o0 An
E(| osp 7 (ML) 1900 wn 277777 MEEEEEE 00 AN
£ o7 777777 EEEEEE 0500 AN 23777777 SN 0300 AN
og [~ 777777 MMM o0 An 24 77777 EEEEEE o0 A
oo 77T MMM 000 A O © 606
10 (- 777777 NN 0500 AN
44 [ 777777 N 0500 AN
12 - 777777 NN 0500 AN
S| T A oo an
| A
B so T o A
- 16 - 777777 EEEEEE 0500 AN
1|0
= @ @ ® @ Send Scheduled Status messages to:
1203 45 67 85 021U e
@ EEEEEEEEEEEEEEEN
T 23 45 67 85 1011 2131 15 1
@ pmmEE e EE R R AR RN
123 45 67 85 1011211
® come: pEEEEEEEREREEEE





1. You can specify up to 24 scheduled commands. Select the triangle next to the number to activate that schedule.

2. The date to enter is in DDMMYY format and the scheduler will check to see if the entered date match todays date to determine if it should execute the programmed command. Placing a “77” in either the “DD”,” MM” or “YY” fields will exclude that field from being checked. It is important that the weekdays should be enabled for the date configured.

3. Weekdays can be selected to execute the command on.

4. The time that the command must be executed can be specified as HHMM

5. The command can be selected from a drop down list. Right click in the CC field for the following selectable list of available commands.

[image: image45.png]AN - Output1 On
AF - Output1 Off
AP - Output1 Puse
BN - Output20n
BF - Output 2 OFF
BP - Output 2 Pulse:
ON- Output30n
CF - Output 3 Off
CP - Output3 Puse
DN - Output 4 On
DF - Qutput 4 OFF
DP - Output 4 Pulse:
EN - Output 5 On
EF - Output 5 OFF
€P - Output5 Pulse
FN - Output6 On
FF - Output6 Off
FP - Output6 Pulse
GN - Output70n
GF - Output 7 Off
GP - Output 7 Pulse

HN - Output 8 On
HE - Outputs Off

HP - Output 8 Puse

MN - Normal Mode

MA-Mode A

ME - Mode B

SM - Send Status Message

LE - D Logger Enable

LD - SD Logger Disable

SH - SMS Hourmeters 14

Sh - SMIS Hourmeters 5-8

C- SMS Counters 14

Sc- SMS Counters 58

L5 - Log Data to SD Card

LG - Log Data to Server via GPRS

ME - EnableModem and SMS Number 16
MD - Disable Modem




Send Scheduled Status messages to:

6. Status: Select the cellphones to send the status messages to.

7. Hour Meter: Select the cellphones to send the hour meter messages to.

8. Counters: Select the cellphones to send the counter messages to.

In the settings as shown above the following will happen:

1: Every Monday to Friday at 08:00 output 1 will be switched ON.

2: Every day at 15:00 output 1 will be switched OFF.

3: On the date 28/06/09 at 21:00 Output 3 will be switched ON.

4: Every Monday, Wednesday and Friday of June 2009 at 18:30 Output 4 will be pulsed. The pulse period will depend on the pulse time specified in the Output 4 configuration page.

5: Every Monday at 08:00 the unit will switch monitoring mode to Mode A.

6: Every Tuesday at 19:00 the unit will switch monitoring mode to Mode N.

7: Every day at 08:00 output 1 will be switched ON.

8: Every Monday at 0800 copy Cellphone 16 to 1

9: Every Tuesday at 0800 copy Cellphone 15to 1

3.7 Monitor mode
The cellcop communicator has a feature to define monitoring of digital and analog inputs based on 3 groups. This is called the monitor mode function. The user can define what is monitored when the unit is switched to a specific monitor mode (N, A or B). Monitor mode can be switched using SMS.
The monitor mode can be used to control what is monitored in a specific mode. As an example you can define Mode N to monitor all inputs, Mode A be to monitor only certain inputs and Mode B to monitor nothing. Then by switching the monitoring mode using the mode switch command u can control the monitoring

Setup the monitor mode using the configuration software. To configure the monitor modes select “Additional functions” tab on the vertical tab menu and then “Monitor Mode” on the horizontal tab menu.

[image: image46.png]Digital Inputs.

Digital Outputs.

Power

System Setup

Testing

(0]

“ Enabe lode N

ModelText MonOn

Allow selected numbers to
]
12 3 45 67
EEEEEN
Inputt On (Contact Closed)
Inputt Off (Contact Open)
Input2 On (Contact Closed)
Input2 Off (Contact Open)
Input3 On (Contact Closed)
Input3 Off (Contact Open)
Inputs On (Contact Closed)
Inputs Off (Contact Open)
Inputs On (Contact Closed)
Inputs Off (Contact Open)
Inputé On (Contact Closed)
Inputé Off (Contact Open)
Input7 On (Contact Closed)
Input? Off (Contact Open)
Inputs On (Contact Closed)
Inputs Off (Contact Open)

\

o E—— -
e cranges

‘SHS containing Control Password allowed to control Montor Hode
N}
59

1011 1213 14 15 16

Enable | Disable Mode Options
Hode N (On) HMode A (Off) Mode B (Skeep)





1. After reading back configuration data from the cellcop the red triangle shows in what mode the board is currently operating.

2. The ModeNText, ModeAText and ModeBText can be specified to be used by SMS commands. So in the example above sending the following will change the current mode accordingly. You can configure your own text.

· *MonOn# – Set to mode N




· *MonOff# – Set to mode A




· *MonSleep# - Set to mode B





3. You can configure what cellphones are allowed to change the mode by SMS. In the example above cellphones 1 – 6 can change the modes.

4. If the “SMS containing Control Password allowed to change monitor mode” is selected you can change the monitor mode from any cellphone as long as the Control Password is also sent. (cpw*12345#)

5. You can select the input states that are monitored in every monitor mode. Above we are monitoring Inputs 1 – 5 the ON and OFF state in Mode N, Input 1- 3 The ON and OFF state in monitor Mode A and only Input 1 the ON and OFF state in monitor Mode B. These setting are exactly the same as the settings on the Inputs, Analogs and power pages and give a convenient overview of monitor mode.

Switch Mode using Scheduled Commands 

See “Scheduled Commands Page”

3.8 System parameters and data call connection
To configure the System parameters and data call connection select “Additional functions” tab on the vertical tab menu and then “System parameters and data call connection” on the horizontal tab menu.

Enter a Site Name here. This site name can be send with messages if so required. Also enter a Programming and Control Password for use when you set parameters or do commands from a cellphone that is not registered with rights on this unit. Use ppw*12345# for programming password and cpw*67890# for command password.

For more detail on data call connection please see section 2.7 (Setting up the configuration software to connect to the communicator using a GSM Modem (Remote configuration)) in this manual
[image: image47.png][ ormentasmessmsonsicomecon | s sy

o SHiStetiame
& Programming Password: Control Password

< 12345 67830

3

Ste Adress / Name / Number:

Remote Cellcop Cellphone Number SETUP MODEM

SIS Message or Hodem Command

Test Message

Connect to Remote Cellcop

Disconnect Remote Cellcop

(Ach'vate Data Cmnmlmi(zti(‘ms] (Send SMS) (Selld Command ]





3.9 GPRS setup
Note that if GPRS is used on the board then server based software is needed for the board to connect to on the internet. This software is called Cellcop Messaging System and is available from your supplier at a cost. Please contact your supplier for more information regarding the server side software

To configure the GPRS select “System Setup” tab on the vertical tab menu and then “GPRS Setup” on the horizontal tab menu.

[image: image48.png]Digital Outputs

Power

‘Additional Functions

Testing

e ————

Enable GPRS Functionality: L @
Own Cellphone Number: -2rs21zs¢se7 (o)
APN: memet [©)] Connection Type:
APN User Name: guest »ce

0] ®

APN Password: st uoe:

» GPRS SVR IP address: v 2 3w @

GPRS SVR Domain Name: ceicop aynans.org

DNS1: 20 212 & 1 ConnectionPort:
DNS2: 2 22 & 1 0000 @
GPRS Login Password: ™
» Seneracknouiedie (1D No Ackncledgement

I CLR SIS Alarmf GPRS succesfully transrited o the server

@

@





1. To use the GPRS functionality, enable it by selecting the check box.
2. Enter the cell number of the SIM used in the device as its used to identify the unit in the field to the server side software that you are connecting to.

3. Enter the APN for the network being used.

4. Enter the user name and password for the APN connection.

5. Select if the connection will be TCP or UDP(Available on request). We usually use TCP.

6. Select if you will use an IP address or Domain Name to connect to. 
7. You can connect to a server by using the Domain Name. You will need to run dynamic DNS functionality at the server side to resolve your dynamic IP address to a constant Domain name hosted at a dynamic domain name service provider.

8. You need to enter Dynamic Name Servers (DNS) IP addresses that can resolve your Dynamic Domain name to the current IP address.

9. The connection port is the port that the server side software has allocated for this unit to connect on.

10. The GPRS password is the password used by the server side software to authenticate the connection being made with the current unit cell phone number.

11.  Server Acknowledge must be switched on if you want the server side software to acknowledge when data is received from the unit.

12. CLR SMS Alarm will only send alarms to server via GPRS if the message is send successfully via GPRS. If not it will also send the alarm via SMS.

3.10 Setting RTC and testing board
Use this page to set the real time clock (RTC) by clicking the Set Date / Time button. The RTC will be set to the Date and time of the PC. Use RTC Data button to read the current RTC setting. Use Get voltages to read the current voltages. Get signal to read the current signal strength. All the replies will be displayed in the memo box.

[image: image49.png]k-

JuseriaiNumeery| | Version | jusDcard jutiotepata|

[o] o]
EE
o2

01

e |





3.11 USB Port

The USB port is used to connect the unit to a PC or Laptop for configuration. The unit will be powered from the USB port. 

[image: image50.png]USB Port
Configuration





Note that the USB driver has to have been installed previously on the PC to connect the Configuration Tool software to the unit. The port made available for the USB interface will also be needed to make the connection.

Please see section 2.6.2 (Setting up the configuration software to connect to the communicator board) in this manual for more information.
3.12 Antenna

Screws onto the antenna plug.

[image: image51.png]



	[image: image52.png]



	[image: image53.png]





3.13 SIM Card
Out

[image: image54.png]



In
[image: image55.png]




4 Controlling the GSM Communicator using a Cellphone
The GSM communicator can be controlled or programmed by sending a SMS to the GSM communicator. 

We divide it in to control commands and programming (Configuration) commands.

If control rights are enabled, then only cellphones with control rights can send a command SMS to the communicator. A custom text can be programmed to be used to switch outputs. 

4.1 SMS Control Commands to communicator

If control rights are enabled, then only those cellphones with control rights can send a command SMS to the communicator.

If “SMS with a control password is selected” then by adding the command password to the start of the SMS message allows you to send control commands from cellphones not defined in the communicator cellphone list.

A custom text can be programmed on the communicator so that if the custom text is send via SMS to the communicator you can use it to switch outputs. 

4.1.1 Custom commands 
Can be issued and must start with a * and end with a #.

Example:

cpw*12345# *out1on# *out2pulse#  *monon#
4.1.2 Status request commands
4.1.2.1 Main status request 
*mainstatus#

4.1.2.2 Hour meter status request   
*hmstatus#

4.1.2.3 Counter Status Request 
 *cntstatus#

Status messages can contain measurement and status information depending on how it was configured.
The following properties exist and should be checked within each message. Not all will be inserted in every message. Process the message and update only properties received.

4.1.2.4 Properties
I:1234567; Inputs

I<x>:z;  x = 1-7, z = 1 or 0;

Mon:Mode; - Text property (Mode is text describing the mode)

Ch:12.5V;
Charger Voltage

Bat:12.2V;
Battery Voltage

O:123--67P; 
Outputs ,  Numbers = on,  - =  Off, P = Pulse

O<x>:z; x = 1-8, z = 1 or 0;

Sig:Value%;

Maintenance Required

H<x>:<Value><Unit>; - Hour Meters, x= 1 to 7

C<x>: <Value><Unit>; - Counters , x= 1 to 7

RT:DD/MM/YY HH:MM:SS; - Real time clock

Maintenance: Required; - Maintenance Status Text field can be any text

Example: “PumpOn” text send to the communicator will switch on Output 1 and “PumpOff” text send to the communicator will switch off Output 1.

.[image: image56.png]Digital Inputs.

Power

dditional Functions

System Setup

Testing

Output 1 Pulse time [Seconds] 1

Enable SHS Text Control
]
on Oution Pulse OutiPusse

oft outtort

Rlow selecied numbers to control OUtpUL 1
|

123 4567 8810

L}

=
=

@ Enabe SMS / Missed Call / Scheduler Control Output 1

11

EnableMissegcal  Voice Fax  Data|  gnapie Scheduler
| ® |

P Puise Output 1 on missed cal

‘Toggle Output 1 on missed cal
Reply with Status SIS

‘SHS containing Control Password alowed to control Output 1

1213 14 15 16





Only Cellphone numbers in position 1 and 2 will be able to switch output1

Adding the password to the SMS from another cellphone will not work as its disabled in the configuration show above

Giving the unit a missed call from cellphone 1 or 2 will pulse output 1 for 1 second.

NB: Outputs can only be controlled with SMS if Output follow is not enabled.

4.2 SMS Programming Commands to communicator

Must contain the programming password. 

ppw*67890# <Programming commands>

Programming commands samples

4.2.1 Set Real Time Clock (RTC)

setrtc*HHMMSSDDMMYYW# - Set RTC

4.2.2 Set hour meter value

sethm<x>*Value # - Set Hour meter. X = 1 to 7
4.2.3 Set counter value

setcnt<x>*Value # - Set Counter value. X = 1 to 7
4.2.4 Set Hour meter Limit for maintenance
setmainhm<x>*Value#

4.2.5 Schedule commands
Shed<x>*<y>DDMMYYHHMMCCWW# x=0 1 to 24, y= 1 or 0

Shed<x>*y# - Switch of/On Logging.

5 Applications

5.1 Alarm System

The CP72u-M has 7 Digital inputs. Inputs are used to monitor a digital state like On and Off or Open and Closed. It can also be used to communicate an Event like Panic triggered or Burglary alarm. By configuring the parameters you can control how the unit utilizes the input. The CP72u-M used a negative trigger to activate and input. A dry contact or switch to GND can be used to activate and deactivate a state of an input. Inputs can also be used as Hour meters and Counters. The following examples show some uses of inputs:

Example 1:

This will typically be for a PIR in the main bedroom wired for a Normally Closed (NC) configuration

[image: image57.png]Digital Outputs.

Power

‘dditional Functions

System Setup

Testing

T e e —— — — —

Tnput 1 On - (Contact Closed) (] enabk tode  (on)_| Enabie iads A (017 || Enabi iade 3 (k.

Delay before tigger 172 sec Unis}: 1 =
S text o send on rigger or CounterHour eter Prefic —
nput 1 0n O
1203 45 6 7 89 101 1213 1415 T
Sl EEEEEEEEEENEEENEN
O
aeRs ony

Anput 1 Off - (Contact Open) I Enabk lode  (On)_| Enabie iads A (017 || Enabl iode 3 (S,

Delay before tigger [172 sec Unis}: 1 =
WS text o send on trigger or CounterHour et Suffc —
Asrm Actvated iai becroom 0
1203 45 67 89 0121 %56
s EEEEEEEENEENEENENN
0
GPRs Oy
Counter 1
() Enable mput 1 Counter  Wutiper 1000000 2 unt Counts
W CountonCose [~ Counton Deayed Close.
Run/Hour Meter 1
() Enable nput 1 Houreter  yutpier 1000000 (25 ne Hours [ Enabie Service SUS Hilt

(RUN on Close)

\ Service Limt 1000

[ 1000 Hours
=

%





Input 1 is setup to send a SMS containing text “Alarm Activated Main Bedroom” to Cell phone number 1 to 16 only if the input 1 was off for half a second and the current Mode is N (normal). 

No message will be sent on input one on. 

Example 2:

This will typically be for a magnetic switch on a door in the house wired for a Normally Closed (NC) configuration

[image: image58.png]Digital Outputs

Power

ddional Functions

System Setup

Testing

— e

Tnput 2 On - (Contact Closed) (] enabk tode  (On)_| Enabie iads A (017 || Enabi iade 3 (Skep.

Deloybefoe rigger [12 sec Unis) 1 =
SHS textto snd ontigger o Countriou et refc o

Input2 0n
12 3 45 6 7 808 1011 1213 14 15 16
sl EEEEEEEEEENEENENENN

GPRS Only.

Tnput 2 Off - (Contact Open) MM Enavk liode  (On)_| Enabie iads A (077 B Enabl iode 5 (Skep.

Delaybetore trgger (12 sec sl 60 =
SHMS text to send on rigger or CounterlHour Meter Suffx o

2 3 45 6 7 89 101 1213 1415 16
S B EEEEEEEEEEENEEN

GPRs Oy
Counter 2

Enable put2 Counter Mutipler 1000000 3] ung Counts

» Couton Cise Counton Deayed Close
Run/Hour Meter 2

Enabie nput2 HourMeter  yutpler 1000000 (5] unk Howrs Enable Serice SUS iz

Ut on Cose)

oo 1o00Heus

L —— -/





The SMS message “Alarm Activated Front Door” will be send if the door is opened for longer than 30 seconds so that the person entering has time to deactivate the alarm on entry.

Door will be monitored in Mode N (Normal) alarm on as well as in Mode B (Sleep). So at night the PIR will be ignored in Mode B but the doors will still be monitored in Mode B. Only in Mode A as with the current operation all the sensors on the alarm will be off.

The Site name will be sent with the SMS message.

Example 3:

This will typically be for a panic button in the house wired for a Normally Open (NO) configuration

[image: image59.png]Power _ Digital Outputs

jonal Functions

System Setup

Testing

wewn [ es | s

Input 1 Input 5

Input 6

Input 7

SMS text to send on trigger or Counter/Hour Meter Pref

s 10 1

\&

Delay before tigger [172 sec Urits]: 1

Service Limt

Tnput 3 On - (Contact Closed) M enavk tode 1 (on) B Enabie iods A (077 Il Enabi iode 5 (Skep.

=
=

Add St Name.
]

1213 14 15 16
EEEEEEEN

Tnput 3 Off - (Contact Open) ] Enabk lode  (On)_| Enabie iads A (017 || Enabl iade 5 (Sep.
Doy before tigger 112 sec Unis] 1 =
SHS textto send on tigge o CounterHour Meter Suffx o
nput3 01t
1203 45 67 89 0121 %56
Sedoo I EEEEEEEEEEEEEER
GPRs Oy
Counter 3
Enable nput3 Courter  Hutipier 1.000000 ] U Counts
» Couton Cise Counton Deayed Close
Run/Hour Meter 3
Enabie nput 3 HourMeter  yutpler 1000000 (5] un Howrs Enable Serice SUS i3
Ut on Cose) o

1000

=
=

-/





The SMS message “Panic Activated” will be send if the panic button is pressed.

Panic button will be monitored in all modes so the alarm can be off (mode A), the panic will still activate.

The Site name will be sent with the SMS message.

5.2 Water Pumped. (Run Meter)
Example 1:

Here we use an input to indicate that a pump pumping water is running or not. The pump is calibrated and we know that it pumps 1.745 cubic liters of water per hour.

[image: image60.png]Digital Outputs

Power

ddional Functions

System Setup

Testing

\ Service Limt 1000

| o — a—

Tnput 1 On - (Contact Closed) M enavk tode  (on) B enabie iods A (077 Il Enabi iade 5 (Skep.

Delay before tigger [172 sec Unis: 2 =
SIS text o send on rigge or CounterHour eter Prefic —
Pump Swiched On O
1203 45 6 7 89 101 1213 1415 T
Sl EEEEEEEEEENEEENEN
L]
aeRs ony

Tnput 1 Off - (Contact Open) I Enavk liode  (on) B Enabie iods A (077 Il Enabi iode 5 (Skep.

Dclay before trgger 12 sec s} 2 =
S text o send on rigge or CounterHour Weler Suffx —
Pump Swiched Off O
1203 45 6 7 89 101 1213 1415 T
ex-EEEEEEEENEEENEEN
n
aeRs ony
Counter 1
() Enable mput 1 Counter  Wutiper 1000000 2 unt Counts
W CountonCose [~ Counton Deayed Close.
Run/Hour Meter 1
I Encie nput 1 HourMeter  yutpler 1745000 (5] unt cuflL | || Enable Serice SWS Hit1

(RUN on Close)

[ 1000 Hours
=

-/





So as per the example above we entered the multiplier of 1.745, the unit as cubic liters and enabled the run Meter on input one by selecting the check box. 

We did not enable a service limit SMS.

No SMS message will be sent on input one on or off, instead the message and Run meter values will be sent via GPRS to a server on the internet where it will be logged to a database. 

The messages will be send and monitored on all modes.

5.3 Water Pumped. (Pulse Counter)

Example 1:

Here we use an input to indicate that a pump pumping water is running or not. The input is connected to a flow meter on the pump and it sends pulses thru as the water is being pumped. The flow meter is calibrated and we know that every 1024 pulses are equal to 1 cubic liter of water pumped.

[image: image61.png]oi

Addional Functions _Power

System Setup

i

Delay before trigger 112 sec Unis}:
‘SHS text o send on trigger or CounteriHour Meter Prefix

Pump Swiched On

12 3 45 6 7 808 1011 1213 14 15 16
sendTo: [][ ][] 0100000000000

L Log change to OnEvent  Log Alarm On Event (Delay)
GPRS Only. 0 0

(Input 1 OFf - (Contact Open) I Ensbe Hode i (on) Bl Ensb oce A (01) B Enabe Hode  (Skep

Delay before trigger [12 sec Units]: 1 5]
‘SHS text 1o send on trigger or CounteriHour Meter Suffoc —

Pump Swiched Off 0

12 3 45 6 7 808 1011 1213 14 15 16
sendTo: [][ ][] 0100000000000

L Log change to OffEvent  Log Alarm Off Event (Delay)
GPRS Only. 0 0

(Counter 1

B ot put 1 Counter Mutipler 0.0005765 (5] Uk cubL |
P CotonCose  [> Counton Delayed Close:

B ————————————————————————————————

Run/Hour Meter 1

et o et 176500 5 ol

Service Limt 1000





So the unit will count a pulse every time the input is activated and the multiplier is 1/1024 cubic liter or 0.0009765625
The unit is cubic liter

No message will be sent on input one on or off as no cell numbers is selected. Values are also not logged or send to a server via GPRS.

5.4 Generator Monitor.
Example 1:

In this example we will monitor a diesel power generator that is used for backup power to the mains supply. The generator is controlled by a PLC and some interfaces are available from the PLC thru relay contacts. 

The PLC provides interfaces for:

· Mains Power On / Off

· Generator Start / Stop

· Generator Over Heat

· Generator Low oil Pressure

We can also interface with a Kw/h meter to monitor how much power is used from mains power. The Kw/h meter provides a pulse output that gives 1 pulse for every 0.001 Kw/h used.

The generator should get a preventive maintenance service every 250 hours of operation.

We would like to set the generator PLC into manual mode / automatic mode remotely via SMS. (This is an input available on the PLC)

We would like to reset the PLC remotely via SMS command. (This is an input available on the PLC)

The following screens will show how we configure the unit to get the functionality as required above.

We load the following cell numbers for the person responsible for the generator (Gen Supervisor) and the service person that does a service on the generator every 250 hours. (Gen Service)

[image: image62.png]/= Cellphone Numbers

1 +27120121234 1 GenSupervisor

2 GenService




So in digital input 1 we monitor the Mains power on and off. We will send the mains power restored / failure messages as shown underneath to the Gen Supervisor if the condition prevails for 5 seconds (Delay trigger of 10). The delay trigger is used to filter out any power dips and only report real failures. 

Messages will also get the site name attached to the back of the message.

The run meter information can be used to see how long we were running on mains and how long we run on generator power.

[image: image63.png]111 1213 14 15 16

ooooood

(Input 1 OFf - (Contact Open) I Ensbe Hode i (on) Bl Ensb oce A (01) B Enabe Hode  (Skep

Delay before tigger [172 sec Unis; 10 =
Ad St Name

‘SHS text 1o send on trigger or CounteriHour Meter Suffoc

111 1213 14 15 16

]
oooooooo

[Counter 1
(] Enable input 1 Counter Hultpier 0001000 5] Uz Kwh
P CotonCose  [> Counton Delayed Close:

Run/Hour Meter 1

I Encie nput 1 Hourbeter  yutipler 1.000000 2] Unit
RUN on Clse)





So in digital input 2 we monitor the Generator Start / Stop. We will send the generator running / rundown messages as shown underneath to the Gen Supervisor if the condition prevails for 5 seconds (Delay trigger of 10). The delay trigger is used to filter out any start failures and only report real events.

The run meter information can be used to see how long we were running on generator and if service intervals have been reached. A Service SMS will be send to the Gen Supervisor and the Gen Service person after every 250 hours is reached.

[image: image64.png]Power

Functions

111 1213 14 15 16

ooooooo

(Input 2 OFf - (Contact Open) M Ensbe Hode i (on) Bl Ensb Hode A (o) B Enabe Hode  (Skep

Delay before tigger [172 sec Unis; 10 =
Ad St Name

|

‘SHS text 1o send on trigger or CounteriHour Meter Suffoc

10 11 1213 14 15 16

ooooooo

(Counter 2

[[] Enable nput2 Counter  Mutipler 1.000000 [ unt Counts|

(RUN on Close)





So in digital input 3 we monitor the generator over heating alarm. We will send the Generator Temperature Over heating / Normal messages as shown underneath to the Gen Supervisor if the condition prevails for 5 seconds (Delay trigger of 10). The delay trigger is used to filter out any power dips and only report real failures. 

Messages will also get the site name attached to the back of the message.

[image: image65.png]Digital Outputs

Power

‘Additional Functions

System Setup

Testing

RS S e S S S S

Tnput 3 On - (Contact Closed) M enavk tode 1 (on) B Enabie iods A (077 Il Enabi iode 5 (Skep.

Delaybetore trgger (172 sec sl 10 =
SHS textto snd ontigger o Countriou et refc o
Generaor Temperature Over heatng n
123 4567 89 WU
SenaTe:
ers ony

Tnput 3 Off - (Contact Open) MM Enabk liode  (on) B Enabie iods A (077 Il Enabi iode 5 (k.

Delay before tgger 12 secUnis]: 10 =
SHS text to send on trigger or CounterHour Meter Suffx o
L. . u

123 45 67 89 0121 %156
SenTo:
GPRs Oy
Counter 3
Enabie nput 3Counter  Hutiler 1000000 [ unt Counts
P Counton Cise Counton Delayed Close

Run/Hour Meter 3

Enabe Input 3 Hour Meter il 1000000 2] Un Hours N Enabic Servie SUS HitS
eceee) @ 1000 Hours

I\ p— v)





So in digital input 4 we monitor the generator oil pressure alarm. We will send the Generator Oil Pressure Low / Normal messages as shown underneath to the Gen Supervisor if the condition prevails for 5 seconds (Delay trigger of 10). The delay trigger is used to filter out any power dips and only report real failures. 

Messages will also get the site name attached to the back of the message.

[image: image66.png]tal Outputs

Power

dditional Functions

System Setup

Testing

— S — ——

Tnput 4 On - (Contact Closed) B enavk tiode 1 (on) B enabie iods A (077 Il Enabi iade 5 (Skep.

=
Delay before trigger 122 sec Units): 0 =
‘SHS text o send on trigger or CounteriHour Meter Prefix adasis
Generator Oil Pressure Low [~
12 3 45 6 7 808 1011 1213 14 15 16
SendTo:
GPRS Only.

Tnput 4 Off - (Contact Open) I Enavk lode 1 (on) B Enabie iods A (077 Il Enabi iode 5 (Skep.

Delay before tigger [172 sec Unis; 10 =
WS text o send on trigger or CounterHour et Suffc o
Generato O Pressure ormal =

123 45 67 89 0N 216
SenTo:
GPRs Oy

Counter 4

Enable nput 4 Cournter  Hutipier 1.000000 [} U Counts

» Couton Cise Counton Deayed Close
Run/Hour Meter 4

Enabie nput  HourMeter  putpler 1000000 (5] unk Howrs Enable Serice SUS il

Gilorein) 000 [= 1000 Hours

\& —— %





So in digital input 5 we will count pulses as given by the Kw/h meter so we can see how much mains power we are using. So we enable the counter and set the multiplier to 0.001 as 100 counts = 1 Kw/h

[image: image67.png]111 1213 14 15 16

ooooooo

(Input 5 OFf - (Contact Open) M nsbe Hode i (on) Bl Ensb Hode A (01) Bl Enabe Hode  (Skep

Delay beforetigger 172 sec Unts]: 1 =
Ad St Name

|

‘SHS text 1o send on trigger or CounteriHour Meter Suffoc

111 1213 14 15 16

ooooooo

(Counter 5

W Enable input S Counter Huipier 0001000 (5] Ung K
P CotonCose  [> Counton Delayed Close:

Run/Hour Meter 5

[ Enale nput S Hour Meter  yutiper 1.000000 (3 Unt. Hours ] Enabe Service SUS HIIS
(RUN on Clse)

Goo = 1000 Hours
Servceimt 1000 3





We will setup output 1 to set the PLC in Automatic mode when the text “AUTOMATIC” is send to it and to run in Manual mode when the text “Manual” is sent to it.

The Generator Supervisor and the Generator Service registered phone numbers is allowed to send commands to control output 1

SMS commands send with a password can be send from other cellphones than the two enabled phones.

[image: image68.png]Digital Inputs

Power

Addtional Functions

System Setup

Testing

Output 1 Pulse time [Seconds] 1

Enable SHS Text Control
]

on I

|
12 3 45 6 7 8
L}

On AUTOMATIC  Puise OutiPuise

s

Rlow selecied numbers to control OUtpUL 1

10

=
=

@ Enabe SMS / Missed Call / Scheduler Control Output 1

11

EnableMissegcal  Voice Fax  Data|  gnapie Scheduler

12

| e
Puise Output 1 on missed call
P Toggle Output 1 on missed call

Reply with Status SIS
]

‘SHS containing Control Password alowed to control Output 1

|
1314 15 16





We will setup output 2 to reset the PLC by sending a pulse out on output 2 for 4 seconds when the text “RESET” is send to it.

The Generator Supervisor and the Generator Service registered phone numbers is allowed to send commands to control output 2
SMS commands send with a password can be send from other cellphones than the two enabled phones.

[image: image69.png]Digital Inputs

Power

‘Additional Functions

System Setup

Testing

Output 2 Pulse time [Seconds] ¢

=
=

@ Enabe SMS /Missed Call / Scheduler Control Output 2

Enable SHS Text Control

EnableMissedcall Vo Fax  Data | copcoger

n u @
on Ou20n Puse RESET P Fuise Output 2 on mised cal
on@EW Toggle Output 2 o missed ca
Reply wih Satus SUS
Alow selected numers foconrol Oyt 2 WS containng Conirl Password alowed to conrolOuput 2
u u

12
L}

3 4567 859

111 1213 18 15 16





6. Troubleshooting
Symptom: Communicator does not send messages.
Possible remedies:
1. Check if there is power on the board. Mains or battery should be available and LED light should come on.
[image: image70.png]



The LED light is on the left as indicated in the photo above

2. Check if the LED is flashing once every 3 seconds. This means the SIM is connected to the network and messages can be sent. Possible problems is that the SIM does not have money available on the account or that the card has not been RICA’ed
3. If The LED is flashing once every 1 second it means that the SIM is not registered on the network. This can be an old SIM that has been deleted of the network or it does not have enough signal strength to connect to the network.
4. If there is power and the SIM is registered on the network with enough money on the account to send messages them make sure the configuration is set up correctly to the right cellphone numbers ant that the input triggers actually occur. 
5. The test page on the configuration software can help a lot as it shows charger voltage and signal strength. You can also see the state of the digital inputs and switch the outputs.
[image: image71.png]k-

JuseriaiNumeery| | Version | jusDcard jutiotepata|

[o] o]
EE
o2

01

e |






IMPORTANT NOTICE





A security system cannot prevent emergencies. It is only intended to alert you and - if programmed - your neighbors and monitoring station of an emergency situation. Security systems are generally very reliable but they may not work under all conditions and they are not a substitute for prudent security practices or life and property insurance. Your security system should be installed and serviced by qualified security professionals who should instruct you on the level of protection that has been provided and on system operations.














10
21

