BLE Key Setup:

Gatemagic:

- Send Command to Gatemagic to Add BLE KEY

Command Format:

CMD: EC*8#

Validation: 1*1#

Key to Confirm: 2*BLE KEY[32 Hex characters]#

Example of Command Text to Set the BLE Key:

Set BLE KEY

EC*8# 1*1# 2*8C6D364D4678E3703468EFB3EC779ED6#

Send Command to Gatemagic by inserting the following into the gm_DataTxTable

- Insert Id of Gatemagic into the gc_AccessPointId Field

- Insert Command Text into the Message Field

- Insert 0 into the Status Field

- Insert Message Expire Date Time into the ExpireDT Field

- Insert 0 into the Priority Field

SQL to send Command:

INSERT INTO gm_DataTxTable (gc_AccessPointId, Message, Status, ExpireDT, Priority)

VALUES (1,’EC*8# 1*1# 2*8C6D364D4678E3703468EFB3EC779ED6#’,0,’2021/03/04 23:00:00’,0)

- Set the BLE module Serial numbers for BLEModule 1 and BLE module 2 by sending command to Gatemagic unit

SQL to Send Add BLE1 Serial Number:

INSERT INTO gm_DataTxTable (gc_AccessPointId, Message, Status, ExpireDT, Priority)

VALUES (1,’ SETBLE1SN*202012030001#’,0,’2021/03/04 23:00:00’,0)

SQL to Send Add BLE2 Serial Number:

INSERT INTO gm_DataTxTable (gc_AccessPointId, Message, Status, ExpireDT, Priority)

VALUES (1,’ SETBLE2SN*202012030002#’,0,’2021/03/04 23:00:00’,0)

Gatemagic BLE/QR Unit via BLE Port

BLESETSN*<Unit Serial Number>#  //202103099004

BSN*202103099004#SSID*GMW202303092005#

*setupwifi#

Gatemagic Wifi / Remote Addon Setup via Server

BLECMD*setserial*<Gatemagic Serial Number>##  //202303092005

- SSID GMW<Serial Number>

OR

BLECMD*setapname*<SSID>##

BLECMD*setappw*<Password>##

BLECMD**setupwifi##

Gatecontroller:

setblekey*Key[32]#

Send Command to Gatecontroller by inserting the following into the gc_DataTx Table

- Insert Id of Gate Controller into the gc_AccessPointId Field

- Insert Command Text into the Message Field

- Insert 0 into the Status Field

- Insert Message Expire Date Time into the ExpireDT Field

- Insert 0 into the Priority Field

SQL to send Command:

INSERT INTO gc_DataTx (gc_AccessPointId, Message, Status, ExpireDT, Priority)

VALUES (1,’setblekey*8C6D364D4678E3703468EFB3EC779ED6#’,0,’2021/03/04 23:00:00’,0)

*getblekey#

INSERT INTO gc_DataTx (gc_AccessPointId, Message, Status, ExpireDT, Priority)

VALUES (1,’*getblekey#’,0,’2021/03/04 23:00:00’,0)

Field – ReplyText Contain the reply of the command and the Key Text

BLECMD*setserial*202110128001##

BLECMD**setupwifi##

SETBLE1SN*202110128001#

SETBLE2SN*202110128002#

EC*8# 1*1# 2*AA44923322E2493C8B00A37DE3E5D3AA#

setupwifiSSID*202007291016#

https://api.qrserver.com/v1/create-qr-code/?data=123456&ecc=H&size=250x250&margin=50

https://goqr.me/api/doc/create-qr-code/